# Photodiode TIA Optimization – Phase 1 (Pangolin Polarity)

Report generated: 2025-09-29 16:46

## Summary

Implemented explicit shot-noise modeling tied to photocurrent using LTspice behavioral current sources. Used a 0 V VSENSE source at the TIA summing node for .noise input reference and an AC=1 A test current (IREF) for AC transimpedance. Performed .noise from 10 Hz to 1 MHz while sweeping α ∈ {0, 0.25, 0.5, 0.75, 1}. Total photocurrent parameter: Itotal = 10 µA. Integrated output-referred noise (NO_RMS) is invariant across α steps, consistent with constant total illumination and sliding-blocker behavior.

## Key LTspice directives

.option plotwinsize=0
.noise V(outputnode) VSENSE dec 200 10 1e6
.ac dec 200 10 10Meg
.meas NOISE NO_PWR INTEG V(onoise)*V(onoise) FROM 10 TO 1e6
.meas NOISE NO_RMS PARAM sqrt(NO_PWR)

## Results (10 Hz – 1 MHz)

| Alpha | NO_RMS (V_rms) |
| --- | --- |
| 0 | 0.001182582844 |
| 0.25 | 0.001182582844 |
| 0.5 | 0.001182582844 |
| 0.75 | 0.001182582844 |
| 1.0 | 0.001182582844 |

## Notes & Next Steps (Phase 2 proposal)

• Compare Pangolin vs Legacy polarity using identical shot-noise setup and AGC conditions.

• Run what‑if sweeps: Itotal scaling (√I), feedback Rf/Cf tradeoffs, input 100 Ω effects, and optional JFET bootstrapping.

• Provide transimpedance and stability (phase margin) checks per configuration.